Tracking Operation Manual

Copyright

This Manual is a property of the Company, and may not be fully or partially reproduced, copied, transcribed or translated in any way without authorization of the Company. Nothing in this Manual shall constitute any guarantee, representation or any other implication in any form. Product specification and other information in this Manual are provided only for reference, and are subject to amendment from time to time without further notification.

Preface

Thank you for choosing the integrated high-definition intelligent teaching tracking camera

This Manual provides a detailed description of how to configure the camera. Before installation and use of the product, please read through this Manual carefully.

Safety Precautions

Before operation, please carefully read and follow the safety precautions set forth in this Manual.

Screenshots contained herein are only for demonstrative purpose, and may differ depending on the specific version.

The Company reserves the right to amend this Manual without any notification or hint, and has made no warranty regarding full correctness of the content herein.

Due to uncertainties that may result from the physical environment or otherwise, some data in practice may deviate from the reference values provided in this Manual. For any doubt or dispute, the final explanation by the Company shall be observed.

 During use of the product, please always follow this Manual. Or otherwise any consequence incurred by failure to use the product following these instructions shall be for your own part. Thank you for your cooperation.

Provisions

Graphs, figures or photos in this Manual are only provided for demonstrative purpose, and may differ depending on the physical object.

Due to uncertainties that may result from the physical environment or otherwise, some data in practice may deviate from the reference values provided in this Manual. For any doubt or dispute, the final explanation by the Company shall be observed.

During use of the product, please observe the instructions in this Manual. You are recommended to use this product following the guidance of a professional.

Table of Contents

1. Network functions	1

2. Configuration process	3

3. Teacher-side configuration	3

3.1 Connecting device	3

3.2 Panoramic lens calibration	3

3.3 Preset position setting	4

3.4 Masking/tracking area setting	4

3.4.1 Tracking area setting	4

3.4.2 Masking area setting	4

3.4.3 Blackboard-writing area setting	5

3.5 Basic parameters configuration	5

3.6 Configuration of advanced parameters	6

3.6.1 Navigation keys	6

3.6.2 Tracking configuration	6

3.6.3 Teacher’s blackboard-writing setting	6

3.6.4 Tracking mode	7

3.6.5 Target action	7

3.6.6 Parameter backup	8

3.6.7 Version information	8

3.7 Enable tracking	8

4. Student-side configuration	8

4.1 Connecting device	8

4.2 Panoramic lens calibration	9

4.3 Preset position setting	9

4.4 Masking area setting	9

4.5 Student area setting	9

4.5 Basic parameters configuration	10

4.6 Configuration of advanced parameters	10

4.6.1 Navigation keys	10

4.6.2 Tracking speed	11

4.6.3 Lens parameters	11

4.6.4 Tracking configuration	11

4.6.5 Target action	11

4.6.6 Close-up hold time	11

4.6.7 Parameter backup	12

4.6.8 Version information	12

4.7 Enable tracking	12

5. Blackboard-side configuration	12

5.1 Connecting camera	12

5.2 Preset position configuration (multiple positions)	13

5.3 Tracking/masking area setting	13

5.3.1 Tracking area setting	13

5.3.2 Masking area setting	13

5.4 Basic parameters configuration	14

5.5 Configuration of advanced parameters	14

5.5.1 Enable blackboard-writing	14

5.5.2 Navigation keys	14

5.5.3 Enable multiple blackboard-writing positions	14

5.5.4 Track loss	14

5.5.5 Parameter backup	14

5.5.6 Version information	15

5.6 Enable tracking	15

6. Instructor-side configuration	15

6.1 Communication Set	15

6.2 Instruction mode	16

6.3 Instructing strategy	16

6.4 Effectiveness of configuration	17

7. Common questions and answers	17

1. Network functions

This Manual mainly introduces the close-up camera network, and contains no information on the panoramic camera network.

            - Enter the network interface

      - View the camera IP:

Connect the network cable to a computer, and use our special camera update software to search for the machine’s IP. (To procure the software, please contact the manufacturer)

      - Enter the camera IP in the browser, and fill the user name: admin and password: admin to enter the network interface.

            - The camera IP can be changed by the following two means:

Method 1: Click [Network configuration] on the webpage to enter the modification interface, make modifications, click Apply, and then restart the camera.

Method 2: Make the change through configuration in our special camera update software,

and then click Set:

            - After change of the camera IP, the camera needs to be restarted, specifically by:

Method 1: Manually power if off and then on for restart

Method 2: Click Restart in configuration of the webpage system (applicable for remote operation)

4. View the software version

Operation prompt

Factory default IP for teacher-side camera: 192.168.100.81/192.

168.100.82

Factory default IP for student-side camera: 192.168.100.91/192.

168.100.92

Network upgrade program downloading instructions

To get the camera upgrade program, please contact the manufacturer.

2. Configuration process

Configuration shall be performed by using the software “tracking configuration tool” from our Company.

The configuration tool interface is shown below:

The teacher-side/student-side camera can be set following the procedures on the left side:

3. Teacher-side configuration

3.1 Connecting device

Connect the teacher-side close-up camera IP and panoramic camera IP; click [Stop tracking] to enter the commissioning state, as shown in the figure.

3.2 Panoramic lens calibration

Tick “Display calibration cross”, and a calibration icon will appear on the preview image of the close-up/panoramic camera; use the tripod head to control upward, downward, leftward and rightward movement of the icon, and make sure the center of the close-up image overlaps with the center of the panoramic image; then click [Calibrate] to calibrate the panoramic lens; click [Reset] to return to the current calibration position, as shown in the figure

3.3 Preset position setting

Set the panoramic position, platform position and close-up blackboard-writing position; [↑], [↓], [←], [→], [Zoom in] and [Zoom out] can be used to adjust the position of the close-up lens; call the panoramic position, platform position and close-up blackboard-writing position to validate effectiveness of the positions, as shown in the figure

3.4 Masking/tracking area setting

3.4.1 Tracking area setting

Tick [Display], and select a tracking area: left-click to determine the starting point of the tracking area, hold down the left mouse button and drag to draw a rectangular tracking area (in the green box)

3.4.2 Masking area setting

To mask interference sources such as the large-size screen in the tracking area, 8 masking areas can be set. Tick [Display], and select a masking area: left-click to determine the starting point of the masking area, hold down the left mouse button and drag to draw a rectangular masking area (in the red box); below the masking area, a part of the tracking area shall be reserved; no masking area shall be set on the left and right boundaries of the tracking area, as shown in the figure

3.4.3 Blackboard-writing area setting

For detection of blackboard-writing, left and right blackboard-writing areas can be set. Tick [Display], and select a blackboard-writing area: left-click to determine the starting point of the blackboard-writing area, hold down the left mouse button and drag to draw a rectangular blackboard-writing area (in the blue box)

3.5 Basic parameters configuration

a) Configure basic parameters, and click [Apply] to effect them

b) Tracking sensitivity: Set the range of action triggering teacher tracking (between 0~4) and the tracking response speed (2 as the default);

c) Size of the tracking image: Set the angle of view for the tracking image of the close-up camera, ranging from 0.5~2.0, Default value 1.0;

d) Distance to the platform: Set the horizontal distance from the teacher-side camera to the platform, ranging from 3.0~20.0 and 6m as the default;

e) Horizontal calibration: Range: -200~+200 (steps), 0.069°/step; moving leftward in case of a negative or rightward in case of a positive; default: 0;

f) Vertical calibration: Range: -200~+200 (steps), 0.069°/step; moving downward in case of a negative or upward in case of a positive; default: 0;

3.6 Configuration of advanced parameters

Click [Unfold], and set advanced parameters.

3.6.1 Navigation keys

Select the tripod head mode or menu mode. In the menu mode, select [Close-up] or [Panoramic] camera; click [Menu] to set the OSD menu of the camera, as shown in Fig. 7 and Fig. 8; in the tripod head mode, the tripod head of [Close-up] camera can be manually controlled.

3.6.2 Tracking configuration

Set a working mode of the tracker from the following five working modes: Startup tracking, auto zoom, no focusing upon tracking, vertical move and throughout tracking; tick the selection to effect it

3.6.3 Teacher’s blackboard-writing setting

Tick Enable blackboard-writing to effect the blackboard-writing function; this function shall be used in combination with the blackboard-writing detection area and preset positions No. 2 and No. 3

Detection time: The shortest time that a blackboard-writing action is responded by tracking after it is detected; default: 2s

Hold time: Duration of holding the blackboard-writing close-up, even though the target has moved out of the detection area during the time; the default duration is 5s.

Missing time: The lapse of time after track loss until execution of the track loss; the default value is 1s.

3.6.4 Tracking mode

Other mode: The close-up only gives a panoramic position

Follow-up mode: As the teacher moves, the close-up will follow the teacher; when the teacher moves quickly out of the close-up or reaches the edge of the close-up, it will switch to a panorama (blackboard-writing function is supported in this mode)

Dual-lens mode: It will show a panorama when the teacher moves, and a close-up when the teacher is still

Mixed mode: When the teacher moves within the reach of close-up, it shows a close-up; when the teacher moves quickly out of the close-up or reaches the edge of the close-up, it will switch to a panorama (blackboard-writing function is supported in this mode)

3.6.5 Target action

a) Track loss action: Set an action of the [Close-up] camera after track loss from: No action, panoramic position (preset position 0), platform position (preset position 1).

Lapse of time: The time that elapses after track loss until execution of the track loss, ranging from 0-15s; the default value is 3s, as shown in Fig. 9.

b) Multi-target action: Set the tracking state in case of multiple targets in the teacher tracking area to Track the moving target or Return to panorama; when multiple targets turn into a single target, it will delay 5s to track the single target, as shown in Fig. 10.

3.6.6 Parameter backup

Parameter backup: It is used to import and export configuration parameters of the close-up camera, and simplify the configuration tasks of the same scenarios, as shown in Fig. 12.

Export: Upon stop of tracking, click [Export] to back up the tracking parameters; click [Open] to view the file path;

Import: Upon stop of tracking, click [Open] to select tracking parameters to be imported; click [Export] and wait for completion;

3.7 Enable tracking

After parameters are configured, click [Apply] to effect the configured parameters; click [Enable tracking] to enter the automatic tracking mode

4. Common questions and answers

Q: After parameters are set and “Apply” is clicked, failure of application is displayed.

A: 1. Incorrect version/no tracking; 2. Firewall active; 3. Multiple configuration tools enabled; 4. Tracking process failure; 5. Conflict of computer IP; 6. Old camera version

Q: Teacher-side/student-side camera fails to perform tracking

A: Check if tracking is enabled, if the panoramic position and close-up position (teacher-side camera) have been set, if a tracking area has been drawn for the teacher-side camera, and if the height parameter of the student-side camera measures correctly

Q: High tracking error rate

A: 1. Teacher-side camera: Mask light sources around

2. Student-side camera: Mask light sources around; enable face detection (face detection shall not be enabled if interleaver mode is required)

Q: The close-up camera version shown at the lower right corner of the tracking configuration tool is 0.0.00

A: 1. The close-up camera has a different protocol/address from the panoramic camera; 2. The tracking process is abnormal and requires upgrading; 3. The serial port connecting the panoramic camera and close-up camera is defective and needs to be repaired at the factory

Q: Vague image and smear occurs when people move in the tracking image of the teacher-side camera

A: Enter the menu, select exposure, change the exposure mode to shutter priority and the shutter speed to 1/100S

Q: The configuration tool indicates an unmatched connection type

A: Make sure the student side connects to a student-side camera and the teacher side connects to a teacher-side camera